ACCRÉDITATION SANS FRONTIÈRES · ISO 9001 READY

Diagnostic Imaging Quality Assurance

Standard Operating Procedure

Approved by Director, [ORGANIZATION NAME] [DIRECTOR NAME] ______________________ Date: __________

| Document Code | SOP-LAB-04-EN |
| --- | --- |
| Title | Diagnostic Imaging Quality Assurance |
| Edition | 2nd Edition — confirmed scope: includes ionizing-radiation imaging (X-ray/CT) |
| Copy # | ___ |
| Effective Date | [EFFECTIVE DATE] |
| Next Review Date | [NEXT REVIEW DATE] |
| Organization | [ORGANIZATION NAME], Georgia |
| Language Pair | Full parallel version of SOP-LAB-04-KA — same content, not a summary |

# Table of Contents

1. Purpose · 2. Scope · 3. Definitions and Abbreviations

4. Open Scope Question · 5. General Provisions · 6. Distribution of Responsibilities

7. Universal Imaging QA (All Modalities) · 8. Ultrasound-Specific QA

9. Conditional: Ionizing Radiation Modalities · 10. Special Considerations

11. Process Effectiveness Indicators (KPI) · 12. Accompanying Documentation

13. Approval Sheet · References

# 1. Purpose

To ensure every diagnostic image produced at [ORGANIZATION NAME] is of consistent, reliable quality, through routine equipment quality assurance appropriate to each imaging modality in use [1–3].

# 2. Scope

This SOP applies to every diagnostic imaging modality operated at [ORGANIZATION NAME]'s diagnostic center.

# 3. Definitions and Abbreviations

•  Phantom — a physical test object with known, reproducible properties, imaged periodically to objectively assess equipment performance rather than relying on subjective impression [1–3]

•  Acceptance testing — the initial, comprehensive performance verification of new imaging equipment, distinct from routine ongoing QA

# 4. Open Scope Question

CONFIRMED WITH [ORGANIZATION NAME]: This diagnostic imaging service includes ionizing-radiation modalities — X-ray and CT — in addition to ultrasound. Section 9 of this SOP is therefore confirmed applicable in full, not conditional. Ionizing-radiation imaging carries real, separate regulatory and radiation-safety obligations that a purely ultrasound-based service does not, and this scope decision is reflected throughout this document.

# 5. General Provisions

Equipment faults in imaging — transducer degradation, image artifacts, physical flaws — are a real, documented issue, and routine QA is disregarded more often than it should be, frequently for cost reasons rather than genuine absence of risk [1]. Acceptance testing of new equipment is the responsibility of a qualified medical physicist or equivalent specialist; routine ongoing QA is the responsibility of a designated, trained technologist [2–3].

# 6. Distribution of Responsibilities

| Task | Director | Radiology/Imaging Lead | Technologist | Quality Manager |
| --- | --- | --- | --- | --- |
| Approval of this SOP | Final |  |  |  |
| Acceptance testing (new equipment) |  | Responsible |  |  |
| Routine QA testing |  |  | Responsible |  |
| QA log review |  | Responsible |  | Informed |
| Out-of-limits equipment investigation |  | Responsible |  | Agreed |

# 7. Universal Imaging QA (All Modalities)

•  Every imaging device is inspected daily for physical damage, cleanliness, and basic function before first use

•  Equipment maintenance and QA testing is logged, including the date, tester, and result of each check

•  Any equipment producing images outside expected quality is taken out of service until the fault is corrected and re-verified

•  Patient identification is confirmed before every imaging study (SOP-PS-01), and images are labeled to prevent mismatch

# 8. Ultrasound-Specific QA

Ultrasound QA guidelines are genuinely lighter than X-ray, CT, or MRI requirements, with emphasis on safety, cleanliness, and basic performance rather than detailed image-resolution testing [3]. Routine ultrasound QA at [ORGANIZATION NAME] includes:

•  Transducer inspection for physical damage or wear before use

•  Basic image performance check using a tissue-mimicking phantom, at a defined periodic interval

•  Transducer disinfection between patients, following the Instrument Reprocessing SOP (SOP-STR-01) for the correct Spaulding classification — transvaginal/transrectal probes require high-level disinfection, not a surface wipe

# 9. Conditional: Ionizing Radiation Modalities

This section is confirmed applicable: [ORGANIZATION NAME] operates ionizing-radiation imaging equipment (Section 4).

•  A documented QA program is required by regulation wherever ionizing-radiation imaging equipment is used on patients, to maintain image quality while minimizing radiation exposure [4–5]

•  Acceptance testing and periodic surveys are performed by a qualified medical physicist, at intervals consistent with accreditation body requirements (e.g., annual physicist survey for mammography-equivalent equipment) [5]

•  Phantom-based testing verifies image uniformity, geometric accuracy, and contrast resolution at defined intervals, with results compared against an established baseline [4]

•  Radiation dose is monitored and recorded per study, with attention to minimizing pediatric dose specifically

# 10. Special Considerations

•  Pediatric imaging studies, where performed, require imaging protocols and (where applicable) dose settings appropriate to smaller body size, not adult defaults

•  Any imaging equipment fault affecting a study already performed triggers review of whether affected prior studies require re-interpretation

# 11. Process Effectiveness Indicators (KPI)

•  Routine QA completion rate, by modality

•  Equipment out-of-service incidents and resolution time

•  Phantom test results, tracked against baseline

# 12. Accompanying Documentation

| Code | Title |
| --- | --- |
| SOP-PS-01 | Patient Identification |
| SOP-STR-01 | Instrument Reprocessing & Sterilization |
| GOV-01 | Nonconformity & Corrective Action |

# 13. Approval Sheet

| Role | Date | Signature |
| --- | --- | --- |
| Prepared by: [RADIOLOGY/IMAGING LEAD NAME] | ___________ | ___________ |
| Agreed by: [QUALITY MANAGER NAME] | ___________ | ___________ |

References

[1] arXiv. A gel wax phantom for performance evaluation in diagnostic ultrasound. 2026.

[2] Phantom systems and methods for diagnostic x-ray equipment (patent filing) — acceptance testing and QA responsibility division.

[3] AAPM. Routine Ultrasound Equipment Tests for Quality Assurance — ACR/AIUM guidelines.

[4] Hobson et al. Using the ACR CT Accreditation Phantom for Routine Image Quality Assurance. Journal of Applied Clinical Medical Physics, 2014.

[5] CASRAI. ACR Accreditation: What It Covers & How It Works. 2026.