ACCRÉDITATION SANS FRONTIÈRES · ISO 9001 READY

Environmental Cleaning & Disinfection

Standard Operating Procedure

Approved by Director, [ORGANIZATION NAME] [DIRECTOR NAME] ______________________ Date: __________

| Document Code | DENT-CC-05-EN |
| --- | --- |
| Title | Environmental Cleaning & Disinfection |
| Edition | 1st Edition |
| Copy # | ___ |
| Effective Date | [EFFECTIVE DATE] |
| Next Review Date | [NEXT REVIEW DATE] |
| Organization | [ORGANIZATION NAME], Georgia |
| Language Pair | Full parallel version of DENT-CC-05-KA — same content, not a summary |

# Table of Contents

1. Purpose · 2. Scope · 3. Definitions and Abbreviations

4. General Provisions · 5. Distribution of Responsibilities

6. The Spaulding Classification · 7. Contact Time — The Real Requirement

8. Terminal Cleaning · 9. Special Considerations

10. Process Effectiveness Indicators (KPI) · 11. Accompanying Documentation

12. Approval Sheet · References

# 1. Purpose

To ensure every surface, device, and patient care space at [ORGANIZATION NAME] is genuinely disinfected — not merely wiped — consistent with CDC HICPAC guidance, incorporated as an enforceable standard under Joint Commission and CMS requirements [1–2].

# 2. Scope

This SOP applies to routine daily cleaning, terminal cleaning after a contaminated procedure or isolation precaution, and reusable device disinfection across [ORGANIZATION NAME].

# 3. Definitions and Abbreviations

•  Contact time (dwell time) — the real, required duration a disinfectant must remain visibly wet on a surface to achieve its labeled kill claim — typically 3–10 minutes, and genuinely distinct from how long it takes to wipe [1]

•  Spaulding classification — the real system classifying reusable devices as critical, semicritical, or noncritical based on patient contamination risk, determining the required level of sterilization or disinfection [2]

# 4. General Provisions

A surface that appears visually clean after being wiped dry in 15 seconds has not necessarily received its labeled contact time — a real, common, and largely invisible failure mode, since the surface looks the same whether disinfection genuinely occurred or not [1].

# 5. Distribution of Responsibilities

| Task | Director | Quality Manager | EVS/Cleaning Staff | Clinical Staff |
| --- | --- | --- | --- | --- |
| Approval of this SOP | Final |  |  |  |
| Cleaning protocol and product selection |  | Responsible |  |  |
| Daily/routine cleaning execution |  |  | Responsible |  |
| Terminal cleaning after contaminated procedure |  |  | Responsible | Responsible |
| Annual competency verification |  | Responsible |  |  |

# 6. The Spaulding Classification

| Spaulding Class | Example | Required Level |
| --- | --- | --- |
| Critical | Enters sterile tissue (e.g., surgical instruments) | Sterilization (see DENT-STR-01) [2] |
| Semicritical | Contacts mucous membranes (e.g., certain probes) | High-level disinfection [2] |
| Noncritical | Contacts intact skin only (e.g., exam tables, stethoscopes) | EPA-registered disinfectant per label directions [2] |

# 7. Contact Time — The Real Requirement

Step 1.  Apply the EPA-registered disinfectant per its label directions — indication, use-dilution, and contact time [2]

Step 2.  Keep the surface visibly wet for the full labeled contact time — typically 3–10 minutes — not wiped dry within seconds [1]

Step 3.  For C. difficile isolation rooms, use an EPA List K sporicidal product — typically 1,000 ppm sodium hypochlorite or a peracetic-acid-based disinfectant, not a standard disinfectant [1]

# 8. Terminal Cleaning

Terminal cleaning follows a defined sequence for any room used for a contaminated procedure or isolation precaution, distinct from routine daily cleaning.

Objective verification (e.g., a UV marker check) genuinely improves terminal clean coverage — real programs using feedback verification improved coverage from 50–70% to 85–95% within 90 days, demonstrating that self-reported completion alone is not a reliable indicator [1].

# 9. Special Considerations

•  Required PPE is matched to the specific isolation precaution type (Contact, Droplet, Airborne) and the cleaning task, not applied uniformly regardless of precaution type [1]

•  Cleaning staff competency is documented at initial training and annually thereafter, including updates specific to any pathogen-specific protocol [1]

# 10. Process Effectiveness Indicators (KPI)

•  Terminal clean coverage, verified objectively, not self-reported alone

•  Contact time adherence, spot-checked during audit

•  Annual competency completion for cleaning staff

# 11. Accompanying Documentation

| Code | Title |
| --- | --- |
| DENT-IPC-01 | Standard Precautions |
| DENT-STR-01 | Instrument Reprocessing & Sterilization |

# 12. Approval Sheet

| Role | Date | Signature |
| --- | --- | --- |
| Prepared by: [QUALITY MANAGER NAME] | ___________ | ___________ |
| Agreed by: [DIRECTOR NAME] | ___________ | ___________ |

References

[1] Opora Supply. CDC HICPAC Environmental Cleaning Guidelines — contact time, C. diff sporicidal requirement, UV marker verification improvement. 2026.

[2] The Joint Commission. Cleaning, disinfection, and sterilization of reusable medical and surgical devices — Spaulding classification, EPA-registered disinfectant requirement. 2023.