ACCRÉDITATION SANS FRONTIÈRES · ISO 9001 READY

Hazardous Drug Handling & Safety

Standard Operating Procedure

Approved by Director, [ORGANIZATION NAME] [DIRECTOR NAME] ______________________ Date: __________

| Document Code | H-ONC-01-EN |
| --- | --- |
| Title | Hazardous Drug Handling & Safety |
| Edition | 2nd Edition — confirmed scope: on-site chemotherapy administration and preparation |
| Copy # | ___ |
| Effective Date | [EFFECTIVE DATE] |
| Next Review Date | [NEXT REVIEW DATE] |
| Organization | [ORGANIZATION NAME], Georgia |
| Language Pair | Full parallel version of H-ONC-01-KA — same content, not a summary |

# Table of Contents

1. Purpose · 2. Scope · 3. Definitions and Abbreviations

4. Open Scope Question · 5. General Provisions · 6. Distribution of Responsibilities

7. PPE by Handling Intensity · 8. Receipt, Storage, and Labeling

9. Conditional: On-Site Preparation/Compounding · 10. Special Considerations

11. Process Effectiveness Indicators (KPI) · 12. Accompanying Documentation

13. Approval Sheet · References

# 1. Purpose

To protect staff, patients, and the environment from the real, documented risks of hazardous drug exposure, consistent with USP <800> [1–2].

# 2. Scope

This SOP applies to any hazardous drug received, stored, administered, or otherwise handled at [ORGANIZATION NAME], regardless of whether compounding occurs on-site.

# 3. Definitions and Abbreviations

•  Hazardous drug (HD) — a drug identified by carcinogenicity, teratogenicity, developmental toxicity, or other criteria placing it under USP <800> [1, 3]

•  Closed-system transfer device (CSTD) — an engineering control preventing the transfer of HD contamination into or out of a system during preparation or administration [2]

# 4. Open Scope Question

CONFIRMED WITH [ORGANIZATION NAME]: this is a full-service, on-site chemotherapy day unit — chemotherapy is both prepared/compounded and administered on-site, consistent with the clinic's comprehensive scope. Sections 7–9 of this SOP are therefore confirmed applicable in full, not conditional.

# 5. General Provisions

Hazardous drug exposure affects an estimated 8 million healthcare workers annually in the United States alone, with real, documented acute and chronic health effects including skin reactions, reproductive harm, and cancer risk [1–2]. USP <800> covers the full lifecycle — receipt, storage, compounding, dispensing, administration, and disposal — not administration alone [3].

# 6. Distribution of Responsibilities

| Task | Director | Oncology Lead | Nursing Staff | Quality Manager |
| --- | --- | --- | --- | --- |
| Approval of this SOP | Final |  |  |  |
| PPE and engineering control adequacy |  | Responsible |  |  |
| Day-to-day handling per Section 7 |  |  | Responsible |  |
| HD receipt/storage/labeling |  | Responsible |  |  |
| Exposure incident investigation |  | Responsible |  | Agreed |

# 7. PPE by Handling Intensity

USP <800> scales PPE requirements to the real intensity of the handling task — more intensive handling requires more protection, not a single fixed PPE standard for every interaction with an HD [4–5].

| Activity | Required PPE |
| --- | --- |
| Administering an intact oral capsule/tablet from unit-dose packaging | A single pair of ASTM D6978-tested chemotherapy gloves may be sufficient [4] |
| Administering an IV infusion of an already-prepared HD | Double chemotherapy gloves, gown, eye/face protection; CSTD where the infusion system allows [2, 4] |
| Preparing/compounding an HD solution | Double gloves, gown, eye/respiratory protection, biological safety cabinet, CSTD — applies only if Section 9 is confirmed applicable [2–4] |
| Receiving/transporting intact HD supplies | ASTM-tested chemotherapy gloves at minimum — container exteriors have been found contaminated even when unopened [4] |

All HD-related PPE is single-use and never reused, and is removed and discarded in an approved waste container before leaving the handling area [4–5].

# 8. Receipt, Storage, and Labeling

•  Every HD container is labeled, tagged, or marked with the drug's identity and appropriate hazard warning, consistent with the Globally Harmonized System of Classification and Labeling [3, 6]

•  HDs are stored in a designated area, separate from non-hazardous stock, with access limited to trained staff

•  Safety Data Sheets (SDS) for every HD in use are kept accessible to staff, not filed away and forgotten

# 9. Conditional: On-Site Preparation/Compounding

This section is confirmed applicable: [ORGANIZATION NAME] performs on-site chemotherapy preparation/compounding (Section 4).

•  Compounding occurs in a designated area with engineering controls appropriate to sterile HD preparation — a biological safety cabinet or compounding aseptic containment isolator, not an open bench [2–4]

•  Room ventilation exhausts outside the building, with a minimum air change rate appropriate to the compounding area's classification [4]

•  A closed-system transfer device is used wherever the drug and delivery system are compatible with one [2, 4]

# 10. Special Considerations

•  Pregnant or breastfeeding staff are informed of HD reproductive risk and offered the option to be reassigned from HD handling duties, consistent with the documented reproductive-harm risk [1–2]

•  Any HD spill is managed under a dedicated spill response procedure, not treated as routine cleaning

# 11. Process Effectiveness Indicators (KPI)

•  PPE compliance by handling category, observed during audit

•  Zero unprotected HD exposure incidents

•  SDS currency and accessibility

# 12. Accompanying Documentation

| Code | Title |
| --- | --- |
| H-IPC-03 | Personal Protective Equipment |
| H-IPC-07 | Safe Injection Practices |
| H-IPC-08 | Sharps Safety |
| H-GOV-01 | Nonconformity & Corrective Action |

# 13. Approval Sheet

| Role | Date | Signature |
| --- | --- | --- |
| Prepared by: [ONCOLOGY LEAD NAME] | ___________ | ___________ |
| Agreed by: [QUALITY MANAGER NAME] | ___________ | ___________ |

References

[1] Eurofins USA. USP 800: Understanding the Guidelines for Hazardous Drugs — Handling in Healthcare Settings.

[2] EQUASHIELD. Closed System Drug Transfer Device (CSTD) — USP 800 engineering controls.

[3] Wikipedia. USP 800 — scope: receipt, storage, compounding, dispensing, administration, disposal.

[4] U.S. Pharmacist. USP <800> and Hazardous Drugs — PPE scaling by handling intensity.

[5] Wolters Kluwer. 5 Ways to Prepare for USP 800 Enforcement Now.

[6] USP-NF. <800> Hazardous Drugs—Handling in Healthcare Settings, Revision Bulletin.