ACCRÉDITATION SANS FRONTIÈRES · ISO 9001 READY

Cytotoxic Waste Disposal

Standard Operating Procedure

Approved by Director, [ORGANIZATION NAME] [DIRECTOR NAME] ______________________ Date: __________

| Document Code | H-ONC-04-EN |
| --- | --- |
| Title | Cytotoxic Waste Disposal |
| Edition | 1st Edition |
| Copy # | ___ |
| Effective Date | [EFFECTIVE DATE] |
| Next Review Date | [NEXT REVIEW DATE] |
| Organization | [ORGANIZATION NAME], Georgia |
| Language Pair | Full parallel version of H-ONC-04-KA — same content, not a summary |

# Table of Contents

1. Purpose · 2. Scope · 3. Definitions and Abbreviations

4. General Provisions · 5. Distribution of Responsibilities

6. Trace vs. Bulk Classification · 7. Waste Streams and Containers

8. Sharps and Spill Materials · 9. Special Considerations

10. Process Effectiveness Indicators (KPI) · 11. Accompanying Documentation

12. Approval Sheet · References

# 1. Purpose

To ensure every item contaminated with a hazardous or cytotoxic drug at [ORGANIZATION NAME] is correctly classified and disposed of through the appropriate real waste stream [1–2].

# 2. Scope

This SOP applies to any waste generated during the receipt, preparation, administration, or spill response involving a hazardous drug at [ORGANIZATION NAME], regardless of whether treatment is administered on-site or the facility only stores/handles such drugs (see H-ONC-01, Section 4).

# 3. Definitions and Abbreviations

•  RCRA empty (trace waste) — an item retaining 3% or less of the original drug volume, which cannot be scraped, poured, or drained to yield additional product [3–4]

•  Bulk waste — an item retaining more than 3% residual drug, or from which the drug can still be scraped, poured, or drained — classified as RCRA hazardous waste [3–4]

# 4. General Provisions

Chemotherapy waste is not a single stream — trace and bulk waste follow genuinely different regulatory pathways, and mixing them is a real, documented compliance failure [1, 3]. The determining test is not appearance but whether residual drug can still be scraped, poured, or drained from the item [4].

# 5. Distribution of Responsibilities

| Task | Director | Oncology Lead | Nursing Staff | Waste Contractor |
| --- | --- | --- | --- | --- |
| Approval of this SOP | Final |  |  |  |
| Trace/bulk classification training |  | Responsible |  |  |
| Day-to-day segregation at point of use |  |  | Responsible |  |
| Bulk hazardous waste collection/transport |  |  |  | Responsible |
| Contractor certification verification |  | Responsible |  |  |

# 6. Trace vs. Bulk Classification

The classification test is not a percentage estimate by eye — it is a real, physical test: if the drug can be scraped, poured, or drained from the item, it is bulk waste, regardless of how little appears to remain [4].

•  Trace (≤ 3% residual, cannot be scraped/poured/drained): empty vials, used gloves/gowns, empty IV tubing, syringes without needles used to administer a completed dose [3–5]

•  Bulk (> 3% residual, or drug can still be scraped/poured/drained): unadministered IV bags/bottles with remaining liquid, syringes with unused drug portions, spill cleanup materials, heavily saturated PPE [3–4]

# 7. Waste Streams and Containers

| Category | Container | Examples | Disposal Route |
| --- | --- | --- | --- |
| Trace chemo waste | Yellow | RCRA-empty items per Section 6 | Incineration only — never autoclaved, never regular trash, never red biohazard bag [3–5] |
| Bulk chemo waste | Black (RCRA hazardous) | Unadministered liquid, unused drug portions, spill materials | Licensed hazardous waste contractor, certified incinerator [1, 4] |
| Regular biohazard | Red | Blood/body-fluid-contaminated items with no chemo contact | Standard regulated medical waste route (does not apply here) |

A real, current list of nine specifically EPA-listed hazardous chemotherapy agents exists (including cyclophosphamide, mitomycin C, and others) — any unused residue or unemptied container of these is bulk hazardous waste without exception [4].

# 8. Sharps and Spill Materials

A standard red biohazard sharps container cannot be used for chemotherapy-contaminated sharps — a separate, dedicated chemotherapy sharps container is required at the point of use [5].

•  Chemotherapy-contaminated needles, syringes, and glass vials go into a dedicated, puncture-resistant chemotherapy sharps container, never the standard sharps container used elsewhere (H-IPC-08)

•  Spill cleanup materials (absorbent pads, contaminated gloves used during cleanup) are bulk waste, going into the black hazardous container, not yellow trace

# 9. Special Considerations

•  The hazardous waste disposal contractor's certifications (EPA, state, local as applicable) are verified and kept on file, consistent with the Supplier Evaluation requirements (Tier 7)

•  General, non-contaminated waste (paper towels, packaging) never goes into the yellow trace container — doing so needlessly increases disposal cost and is itself a documented compliance issue

# 10. Process Effectiveness Indicators (KPI)

•  Waste stream segregation accuracy, verified during audit

•  Zero instances of bulk waste found in the trace container, or vice versa

•  Hazardous waste contractor certification currency

# 11. Accompanying Documentation

| Code | Title |
| --- | --- |
| H-ONC-01 | Hazardous Drug Handling & Safety |
| H-ONC-02 | Extravasation Management |
| H-IPC-08 | Sharps Safety |

# 12. Approval Sheet

| Role | Date | Signature |
| --- | --- | --- |
| Prepared by: [ONCOLOGY LEAD NAME] | ___________ | ___________ |
| Agreed by: [QUALITY MANAGER NAME] | ___________ | ___________ |

References

[1] Daniels Health. How to Dispose of Chemotherapy Waste — trace vs. bulk, EPA-listed hazardous drugs. 2026.

[2] Medical Waste Pros. Chemotherapy Waste Management — bulk hazardous waste disposal requirements.

[3] TriHaz Solutions. Yellow Bin Waste in Hospital: Chemotherapy Waste Disposal Guide.

[4] HMWS Corp. How to Safely Dispose of Chemotherapy Waste: Full Protocol Guide — RCRA-listed agents. 2026.

[5] Sharps Disposal Containers. Trace Chemotherapy Waste Disposal: Healthcare Facilities Guide — dedicated chemo sharps containers. 2026.